Edge / Surround Design

Type Reference  ·  Geometry  ·  SolidWorks Simulation Targets

Calibration Handoff Document

# Purpose  &  Scope

This document defines each surround / edge type implemented in EdgeGeometry.vb. For each type it specifies the cross-section geometry, the compliance physics model, the key parameters to vary in SolidWorks FEA, and what output quantities the simulation should match to calibrate the analytical model.

Note: Edge design is now treated as an independent module, fully decoupled from cone and dustcap geometry. The edge attaches to the cone via the Inner Flange L dimension (slant length along cone surface) but the roll body itself is defined entirely by Active OD, Active Width, and the type-specific roll parameters.

## Simulation Calibration Strategy

- Run static axial deflection in SW for each type at small displacement (linear regime)

- Extract reaction force at inner attach for a known displacement → compute Kms = F/x

- Compare to EdgeGeometry.vb Kms output with identical geometry and material

- Adjust C_break_pos / C_break_neg by matching SW large-displacement Kms(x) curve shape

- Target match within ±5% on Kms at x=0 and within ±10% on X_break

# Common Geometry  —  All Types

Every surround type shares the same annular coordinate system:

| Dimension | Symbol | Definition |
| --- | --- | --- |
| Active OD | D_active | Outer diameter of roll body. User input. Independent of cone OD. |
| Active Width | W | Radial width of roll body. R_inner = Active_OD/2 − W |
| Inner Roll ID | D_inner | = Active_OD − 2×W. Inner attach circle of roll. |
| Inner Flange L | L_f | Slant length of glue tab along cone surface. Free section if L_f > radial gap. |
| Outer Flange W | W_f | Flat sleeve outside Active OD, clamped to basket. |
| Frame ID | D_frame | = Active_OD + 2×Outer_Flange_W. Fully derived. |
| Thickness | T | Uniform shell thickness. Same for roll + flanges. |

For SW models: fix the outer flange face (basket clamp). Apply axial displacement to the inner attach ring. Measure reaction force. Use shell elements with the material Young’s modulus and Poisson ratio defined per type.

# Surround Type Definitions

Type 0  —  Half Roll    Most common woofer / subwoofer surround

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll T E Nu | Single semicircle arc Chord = 2×R_roll = Active Width Arch height = R_roll above baseline Centroid at R_inner + R_roll | Castigliano curved beam theta = pi (full half-circle) Cms = 3R³π / (E T³ × 2π R_c) Kms = 1 / Cms Asymmetric: roll opens fwd, closes rwd | Match Kms at x=0 Match X_break_pos (roll unrolls) Match X_break_neg (roll buckles) Plot Kms(x) to +/− Xmax |

Notes:  Constraint: 2×R_roll = Active Width (exactly). If R_roll < W/2 a flat section remains at each end — model accordingly.

Type 1  —  Double Roll (M-profile)    Series compliance, extended Xmax

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll (inner arc) R_roll2 (outer arc) T E | Two consecutive half-arcs Inner arc radius R_roll Outer arc radius R_roll2 Total width = 2(R_roll+R_roll2) Arcs curve in opposite directions (M shape) | Series Cms: 1/Kms_total = 1/Kms1 + 1/Kms2 Each arc: Cms_i = Castigliano at its centroid Xmax ≈ double that of equivalent single roll Asymmetry: inner arc drives rearward break | Simulate each arc independently first Verify series compliance formula Check transition zone between arcs Calibrate C_break for each arc separately |

Notes:  The two arcs may have different radii (asymmetric M). Inner arc centroid at R_inner+R_roll, outer at R_inner+2R_roll+R_roll2.

Type 2  —  Uneven Roll    Asymmetric mounting height

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll Offset_z (height diff) T E | Single half-roll arc Inner and outer flanges at different axial heights Offset_z = axial height difference mm Effective radius R_eff = sqrt(R² + (dz/2)²) | R_eff replaces R_roll in Castigliano Accounts for geometric stiffening from tilt Cms = 3 R_eff³ pi / (E T³ 2pi R_c) Asymmetry enhanced by axial offset | Apply offset_z as axial misalignment of inner vs outer attach rings Measure Kms vs symmetric (offset=0) baseline Quantify R_eff correction factor Check that SW Kms matches R_eff formula |

Notes:  Used when cone and basket are at different axial heights. Offset_z is signed: positive = inner flange below outer.

Type 3  —  Flat / Cloth    Tweeter and compression driver surrounds

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| T E Nu (critical) | Flat annular ring No arch — purely planar Inner radius = R_inner, outer = R_outer Width W = Active Width | Annular thin-plate bending Kms = E T³ / (A × f(Nu, R_inner/R_outer)) Nu has significant effect — use 0.28–0.35 for cloth Symmetric Kms(x) — no roll geometry | Model as annular plate with fixed outer edge Vary Nu from 0.28 to 0.45, plot Kms sensitivity Compare to analytical formula for annular plate Check buckling mode at large displacement |

Notes:  Cloth surrounds are orthotropic in reality. Use warp/weft E values if available. The isotropic model is a first approximation.

Type 4  —  Accordion    High-excursion PA and pro audio

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| N_pleats (count) H_pleat (half-height) T E | N identical pleats across Active Width Each pleat = two half-arcs of radius H_pleat Total compliance = N × single pleat Cms Pleats spaced evenly: pitch = W/(N+1) | Each pleat modelled as Castigliano arch Series: N pleats in axial series Cms_total = N × Cms_pleat Symmetric: pleat geometry is symmetric | Model single pleat with periodic BC Scale Kms by 1/N to get total Vary N at fixed W: verify 1/N scaling Check pleat contact at large -x displacement |

Notes:  Key failure mode: inner pleat-to-pleat contact at large rearward displacement. SW contact simulation recommended at X = 0.8×Xmax.

Type 5  —  Tangential    Vintage hi-fi, low distortion

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll T E | Roll axis tangential to circumference Not radial — roll corrugation runs circumferentially Equivalent radial width = 2R_roll / cos(30°) Centroid at R_inner + R_roll / cos(30°) | R_eff = R_roll / cos(30°) = 1.155 × R_roll Castigliano with R_eff Lower Kms than equivalent radial half-roll Very low 2nd harmonic distortion | Model full 3D corrugation — not axisymmetric Use sector model with periodic BC at 30° Compare sector Kms × (360/30) to analytical Verify R_eff correction factor of 1.155 |

Notes:  This type requires a 3D sector FEA model, not axisymmetric. The 30° tangential angle is the standard historical value.

Type 6  —  Inverted Roll    Subwoofer, dome outward from basket

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll T E | Half-roll but arches toward basket (downward in cross-section) Centroid at R_outer − R_roll (outer side) Inner attach at cone OD, outer attach at basket Arch opens toward motor — not toward listener | Castigliano with centroid at R_outer − R_roll C_break_neg > C_break_pos (geometry reversed from Type 0) Forward stroke: roll tightens → breaks sooner Rearward stroke: roll opens → breaks later | Same axisymmetric model as Type 0 Swap sign convention on C_break Verify C_break_neg > C_break_pos in calibration Check clearance between dome and basket at +Xmax |

Notes:  Critical clearance check: inverted roll dome must not contact basket at maximum forward excursion. Include in SW assembly model.

Type 7  —  W-Roll    Extended Xmax, automotive subwoofers

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_w1 (inner) R_w2 (middle) R_w3 (outer) T E | Three consecutive half-arcs, alternating direction Arc 1 (R_w1): up. Arc 2 (R_w2): down. Arc 3 (R_w3): up Total width = 2(R_w1 + R_w2 + R_w3) Arcs may have unequal radii | Series Cms: 1/K_total = 1/K1 + 1/K2 + 1/K3 Each arc Castigliano at its own centroid Xmax approximately triple equivalent single roll Inner and outer arcs drive asymmetry | Simulate each arc in isolation first Verify series formula by progressive assembly Plot Kms(x) for asymmetric (R_w1≠R_w3) case Identify which arc buckles first at Xmax |

Notes:  With equal radii (R_w1=R_w2=R_w3=R), compliance = 3× single roll. Asymmetric radii allow tuning Xmax_pos vs Xmax_neg independently.

Type 8  —  Ribbed Half-Roll    Stiffened woofer / subwoofer

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll N_ribs H_rib (rib height) T E | Standard half-roll arc (Type 0 base) Radial ribs molded into roll surface Ribs run from inner to outer attach N_ribs evenly spaced around circumference | Kms = Kms_base × (1 + rib_stiffness_factor) Rib factor depends on H_rib, N_ribs, T Stiffness increase roughly proportional to (T+H_rib)³/T³ Asymmetry same as Type 0 | Model sector with and without rib Extract rib stiffness factor as function of H_rib/T Calibrate rib factor equation in EdgeGeometry.vb Vary N_ribs: confirm linear scaling assumption |

Notes:  Ribs primarily increase Kms without changing Xmax. H_rib/T ratio is the key parameter. SW sector model with one rib is sufficient if N_ribs > 4.

Type 9  —  Lip / Short Roll    Midrange drivers, small surrounds

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_roll T E | Partial arc — less than pi (short of a full half-roll) Roll blends into membrane-like flat section Effective theta < pi Used when Active Width is small relative to R_roll | Cms = blend of Castigliano arch + membrane tension At small theta: membrane-dominated (stiffer) At theta near pi: arch-dominated (softer) Blend factor varies continuously with theta | Model at theta = pi/2, 2pi/3, 5pi/6, pi Extract Cms(theta) curve from FEA Fit blend equation in EdgeGeometry.vb Verify transition from membrane to arch regime |

Notes:  This type is common in 3”–5” midrange drivers where shallow profile is needed. The short roll also reduces surround mass for improved HF extension.

Type 10  —  Tri-Radius    Audiophile woofers, complex profile

| Parameters | Geometry | Physics model | SW sim targets |
| --- | --- | --- | --- |
| R_tri_inner (inner flank) R_tri_crown (crown) R_tri_outer (outer flank) T E | Three mutually tangent arcs, all tangent to flat flanges Inner flank C3 anchored at inner attach Outer flank C1 anchored at outer attach Crown C2 solved from mutual tangency equations (closed-form) J1/J2 = tangency join points | Series Cms from three arcs in series Each arc Castigliano at its centroid radius Crown arc (R_tri_crown) dominates compliance Inner/outer flank arcs provide smooth transitions | Full profile FEA is essential — simple series formula is approximate Build exact profile from tangency solver output (J1x, J1y, J2x, J2y) Compare Kms for symmetric (inner=outer) vs asymmetric Vary R_tri_crown: verify quadratic Cms scaling |

Notes:  The closed-form tangency solver (TriRadius_Solve in EdgeGeometry.vb) outputs exact circle centers C1/C2/C3 and join points J1/J2. Use these coordinates directly to build the SW sketch — do not redraw by eye.

# Material Reference for SW Models

Use isotropic elastic material in SW for all types. Hyperelastic (Mooney-Rivlin) is more accurate for rubber but requires additional material constants. Start with linear elastic for calibration.

| Material | E (MPa) | Nu | Rho (g/cm³) | Primary types | Eta | T range mm |
| --- | --- | --- | --- | --- | --- | --- |
| Foam polyether | 1.2 | 0.46 | 0.080 | 0,1,2,7,8 | 0.28 | 1.2–2.0 |
| Foam polyester | 2.0 | 0.46 | 0.090 | 0,4,8 | 0.20 | 1.5–2.5 |
| Rubber NBR | 5.0 | 0.49 | 1.200 | 0,1,6,8,9 | 0.20 | 0.8–2.0 |
| Rubber SBR | 4.0 | 0.49 | 1.100 | 6,7,8 | 0.24 | 1.0–2.5 |
| Cloth treated | 1800 | 0.30 | 0.300 | 3 | 0.08 | 0.3–0.6 |
| Santoprene/TPE | 2.5 | 0.46 | 0.100 | 0,9 | 0.25 | 1.0–2.0 |

Note: E values are quasi-static. Dynamic E (at acoustic frequencies) is typically 1.2×1.5× the static value for foam. Use static E for displacement / Kms calibration. Use dynamic E for modal analysis.

# SW Calibration Protocol

## Step 1  —  Geometry Setup

- Build axisymmetric 2D sketch from cross-section dimensions

- For Type 5 (tangential): use 3D 30° sector model with symmetric BCs

- For Type 10 (tri-radius): import J1/J2 coordinates from EdgeGeometry.SolveTriRadius() output

- Shell element thickness = T_surround

- Fix all DOF at outer flange face (basket side)

- Apply axial displacement at inner attach ring edge

## Step 2  —  Linear Kms Calibration

- Apply x = 0.1 mm axial displacement (small, linear regime)

- Compute Kms_SW = Reaction_Force / 0.1 mm

- Set Active_Width, R_roll, T, E to match SW model exactly in EdgeGeometry.vb

- Compare Kms_SW to EdgeGeometry result. Target: within ±5%

- If outside 5%: check centroid radius formula and theta assumption

## Step 3  —  Nonlinear Xbreak Calibration

- Run nonlinear static with displacement 0 → Xmax in 20 steps, forward and rearward

- Plot Kms(x) = F(x) / x for each step

- Identify X_break as inflection point where Kms begins rising

- Adjust C_break_pos: X_break_pos = C_break_pos × 2 × R_roll

- Adjust C_break_neg similarly

- Target: inflection point matches within ±10% in both directions

## Step 4  —  Outputs to Record per Type

| Output | Unit | Description |
| --- | --- | --- |
| Kms_SW | N/mm | Linear stiffness at x=0 |
| X_break_pos | mm | Forward break point (inflection in Kms(x)) |
| X_break_neg | mm | Rearward break point |
| Kms_ratio | — | Kms at Xmax / Kms at 0 (nonlinearity index) |
| C_break_pos | — | Calibrated = X_break_pos / (2 × R_roll) |
| C_break_neg | — | Calibrated = X_break_neg / (2 × R_roll) |
| Max stress | MPa | Peak principal stress at Xmax (fatigue check) |

EdgeCone Designer  ·  Surround Type Reference  ·  SolidWorks Calibration Handoff